| United States Environmental Protection Agency | Office of Pollution Prevention and Toxics Washington, DC 20460 | 740B19039 January 2020 |
| --- | --- | --- |

TOXICS RELEASE INVENTORY

Form R and Form A Certification Statement Reporting Codes and Instructions for Reporting Metals

Section 313 of the Emergency Planning and Community Right-to-Know Act of 1986 (EPCRA) requires certain facilities manufacturing, processing, or otherwise using listed toxic chemicals to report the annual quantity of such chemicals entering each environmental medium. Such facilities must also report pollution prevention and recycling data for such chemicals, pursuant to section 6607 of the Pollution Prevention Act, 42 U.S.C. 13106. EPCRA section 313 is also known as the Toxics Release Inventory (TRI).

CONTENTS

Chapter 1 Form R and Form A Certification Statement Reporting Codes	1

Revision Codes	1

Withdrawal Codes	1

EPCRA Section 313 Chemical Category Codes	1

Section 3. Activities and Uses of the EPCRA Section 313 Chemical at the Facility	1

Section 4. Maximum Amount of the Toxic Chemical On-Site at Any Time During the Calendar Year	2

Section 5. Quantity of the Toxic Chemical Entering Each Environmental Medium On-Site and Section 6 Transfer(s) of the Toxic Chemical in Wastes to Off-Site Locations	2

Section 6.1 Discharges to Publicly Owned Treatment Works: Disposal / Treatment Codes (POTW)	3

Section 6.2 Transfers to Other Off-Site Locations: Type of Waste Disposal/Treatment/Energy Recovery/Recycling	3

Section 7A: On-Site Waste Treatment Methods and Efficiency	5

Section 7B: On-Site Energy Recovery Processes	5

Section 7C: On-Site Recycling Processes	5

Section 8.10 Source Reduction Activity Codes	6

Section 8.11 Optional Pollution Prevention Information	7

Chapter 2 Reporting the Waste Management of Metals	8

Section 5.3 Discharges to Receiving Streams or Water Bodies and Section 6.1 Discharges to Publicly Owned Treatment Works	8

Section 6.2 Transfers to Other Off-Site Locations	9

Section 7A: On-Site Waste Treatment Methods and Efficiency	9

Section 7B: On-Site Energy Recovery Processes	10

Section 7C: On-Site Recycling Processes	10

Section 8. Source Reduction and Recycling Activities	10

DISCLAIMER

This guidance document is intended to assist industry by summarizing TRI Form R and Form A Certification Statement reporting codes and specific instructions for reporting metals. These recommendations do not supersede any statutory or regulatory requirements, are subject to change, and are not independently binding on either EPA or covered facilities. Additionally, if a conflict exists between guidance on this site and the statutory or regulatory requirements, the conflict must be resolved in favor of the statute or regulation.

Although EPA encourages industry to consider these recommendations, in reviewing this document, industry should be aware that these recommendations were developed to address common circumstances at typical facilities. Facilities are encouraged to contact the Agency with any additional or clarifying questions about the recommendations in this document, or if the facility believes that EPA has incorrectly characterized a particular process or recommendation.

Additional guidance documents, including industry specific and chemical specific guidance documents, are also available on TRI’s GuideME website: https://ofmpub.epa.gov/apex/guideme_ext/f?p=guideme:gd-list

OVERVIEW

This document is intended to assist establishments and facilities with summarizing TRI Form R and Form A Certification Statement reporting codes and specific instructions for reporting metals. The EPCRA section 313 program is commonly referred to as the Toxic Chemical Release Inventory (TRI). For background on the TRI program, resources for determining whether a facility must report, and reporting requirements, please refer to the current TRI Reporting Forms and Instructions, also available on GuideME.

# Form R and Form A Certification Statement Reporting Codes

This chapter summarizes reporting codes currently used for the TRI Form R and Form A Certification Statement (OMB Number: 2025-0009; Approval expires: 10/31/2021). Reporting codes are grouped by the appropriate Form R section. Codes no longer in use are listed as retired codes.

## Revision Codes

RR1	New Monitoring Data

RR2	New Emission Factor(s)

RR3	New Chemical Concentration Data

RR4	Recalculation(s)

RR5	Other Reason(s)

## Withdrawal Codes

WT1	Did not meet the reporting threshold for manufacturing, processing, or otherwise use

WT2	Did not meet the reporting threshold for number of employees

WT3	Not in a covered NAICS Code

WO1	Other reason(s)

## EPCRA Section 313 Chemical Category Codes

N010	Antimony compounds

N020	Arsenic compounds

N040	Barium compounds

N050	Beryllium compounds

N078	Cadmium compounds

N084	Chlorophenols

N090	Chromium compounds

N096	Cobalt compounds

N100	Copper compounds

N106	Cyanide compounds

N120	Diisocyanates

N150	Dioxin and dioxin-like compounds

N171	Ethylenebisdithiocarbamic acid, salts and esters (EBDCs)

N230	Certain glycol ethers

N270	Hexabromocyclododecane

N420	Lead compounds

N450	Manganese compounds

N458	Mercury compounds

N495	Nickel compounds

N503	Nicotine and salts

N511	Nitrate compounds

N530	Nonylphenol

N535	Nonylphenol ethoxylates

N575	Polybrominated biphenyls (PBBs)

N583	Polychlorinated alkanes

N590	Polycyclic aromatic compounds

N725	Selenium compounds

N740	Silver compounds

N746	Strychnine and salts

N760	Thallium compounds

N770	Vanadium compounds

N874	Warfarin and salts

N982	Zinc compounds

## Section 3. Activities and Uses of the EPCRA Section 313 Chemical at the Facility

### Section 3.2 Process Sub-Use Codes

3.2.a: As a Reactant

P101	Feedstocks

P102	Raw materials

P103	Intermediates

P104	Initiators

P199	Other

3.2.b: As a formulation component

P201	Additives

P202	Dyes

P203	Reaction diluents

P204	Initiators

P205	Solvents

P206	Inhibitors

P207	Emulsifiers

P208	Surfactants

P209	Lubricants

P210	Flame retardants

P211	Rheological modifiers

P299	Other

### Section 3.3 Otherwise Use Sub-Use Codes

3.3.a: As a chemical processing aid

Z101	Process solvents

Z102	Catalysts

Z103	Inhibitors

Z104	Initiators

Z105	Reaction terminators

Z106	Solution buffers

Z199	Other

3.3.b: As a manufacturing aid

Z201	Process lubricants

Z202	Metalworking fluids

Z203	Coolants

Z204	Refrigerants

Z205	Hydraulic fluids

Z299	Other

3.3.c: Ancillary or other use

Z301	Cleanser

Z302	Degreaser

Z303	Lubricant

Z304	Fuel

Z305	Flame retardant

Z306	Waste treatment

Z307	Water treatment

Z308	Construction materials

Z399	Other

## Section 4. Maximum Amount of the Toxic Chemical On-Site at Any Time During the Calendar Year

| Range Code | From | To |
| --- | --- | --- |
| 01 | 0 | 99 |
| 02 | 100 | 999 |
| 03 | 1,000 | 9,999 |
| 04 | 10,000 | 99,999 |
| 05 | 100,000 | 999,999 |
| 06 | 1,000,000 | 9,999,999 |
| 07 | 10,000,000 | 49,999,999 |
| 08 | 50,000,000 | 99,999,999 |
| 09 | 100,000,000 | 499,999,999 |
| 10 | 500,000,000 | 999,999,999 |
| 11 | 1 billion | More than 1 billion |

## Section 5. Quantity of the Toxic Chemical Entering Each Environmental Medium On-Site and Section 6 Transfer(s) of the Toxic Chemical in Wastes to Off-Site Locations

### Range Codes for Non-PBT Chemicals

| Range Code | From |
| --- | --- |
| A | 1-10 |
| B | 11-499 |
| C | 500-999 |

### Basis of Estimate

M1	Estimate is based on continuous monitoring data or measurements for the EPCRA section 313 chemical.

M2	Estimate is based on periodic or random monitoring data or measurements for the EPCRA section 313 chemical.

C	Estimate is based on mass balance calculations, such as calculation of the amount of the EPCRA section 313 chemical in streams entering and leaving process equipment.

E1	Estimate is based on published emission factors, such as those relating release quantity to through-put or equipment type (e.g., air emission factors). This may include emissions factors in a trade associations publication or AP-42.

E2	Estimate is based on site-specific emission factors, such as those relating release quantity to through-put or equipment type (e.g., air emission factors). This may include emissions factors that are developed for a specific piece of equipment and that consider climate conditions on-site.

O	Estimate is based on other approaches such as engineering calculations (e.g., estimating volatilization using published mathematical formulas) or best engineering judgment. This would include applying an estimated removal efficiency to a waste stream, even if the composition of the stream before treatment was fully identified through monitoring data.

## Section 6.1 Discharges to Publicly Owned Treatment Works: Disposal / Treatment Codes (POTW)

Disposal

P30	Discharged to Water Stream

P31	Discharged to Other Activities

P32	Released to Air

P33	Sludge to disposal

P34	Metals and metal compounds only – Sludge to incineration

P35	Sludge to agricultural applications

P36	Other or Unknown Disposal

Treatment

P37	Other or Unknown Treatment

P38	Sludge to incineration

P39	Experimental and Estimated Treatment Data (TRI provided)

## Section 6.2 Transfers to Other Off-Site Locations: Type of Waste Disposal/Treatment/Energy Recovery/Recycling

Disposal

M10	Storage Only

M41	Solidification/Stabilization - Metals and Metal Category Compounds only

M62	Wastewater Treatment (Excluding POTW) - Metals and Metal Category Compounds only

M64	Other Landfills

M65	RCRA Subtitle C Landfills

M66	Subtitle C Surface Impoundment

M67	Other Surface Impoundments

M73	Land Treatment

M79	Other Land Disposal

M81	Underground Injection to Class I Wells

M82	Underground Injection to Class II-V Wells

M90	Other Off-Site Management

M94	Transfer to Waste Broker – Disposal

M99	Management Method Unknown

Treatment

M40	Solidification/Stabilization

M50	Incineration/Thermal Treatment

M54	Incineration/Insignificant Fuel Value

M61	Wastewater Treatment (Excluding POTW)

M69	Other Waste Treatment

M95	Transfer to Waste Broker - Waste Treatment

Energy Recovery

M56	Energy Recovery

M92	Transfer to Waste Broker - Energy Recovery

Recycling

M20	Solvents/Organics Recovery

M24	Metals Recovery

M26	Other Reuse or Recovery

M28	Acid Regeneration

M93	Transfer to Waste Broker - Recycling

### Retired Codes

M63	Surface impoundment (retired effective RY2003)

M71	Underground injection (retired effective RY2003)

M72	Landfill/Disposal surface impoundment (retired effective RY2002)

M91	Transfer to waste broker (retired effective RY1991)

## Section 7A: On-Site Waste Treatment Methods and Efficiency

### General Waste Stream

A	Gaseous (gases, vapors, airborne particulates)

W	Wastewater (aqueous waste)

L	Liquid waste streams (non-aqueous waste)

S	Solid waste streams (including sludges and slurries)

### Waste Treatment Methods

Air Emissions Treatment

A01	Flare

A02	Condenser

A03	Scrubber

A04	Absorber

A05	Electrostatic Precipitator

A06	Mechanical Separation

A07	Other Air Emission Treatment

Chemical Treatment

H040	Incineration--thermal destruction other than use as a fuel

H071	Chemical reduction with or without precipitation

H073	Cyanide destruction with or without precipitation

H075	Chemical oxidation

H076	Wet air oxidation

H077	Other chemical precipitation with or without pre-treatment

Biological Treatment

H081	Biological treatment with or without precipitation

Physical Treatment

H082	Adsorption

H083	Air or steam stripping

H101	Sludge treatment and/or dewatering

H103	Absorption

H111	Stabilization or chemical fixation prior to disposal

H112	Macro-encapsulation prior to disposal

H121	Neutralization

H122	Evaporation

H123	Settling or clarification

H124	Phase separation

H129	Other treatment

## Section 7B: On-Site Energy Recovery Processes

U01	Industrial Kiln

U02	Industrial Furnace

U03	Industrial Boiler

## Section 7C: On-Site Recycling Processes

H10	Metal recovery (by retorting, smelting, or chemical or physical extraction)

H20	Solvent recovery (including distillation, evaporation, fractionation or extraction)

H39	Other recovery or reclamation for reuse (including acid regeneration or other chemical reaction process)

## Section 8.10 Source Reduction Activity Codes

### Source Reduction Activity Codes

Good Operating Practices

W13	Improved maintenance scheduling, record keeping, or procedures

W14	Changed production schedule to minimize equipment and feedstock changeovers

W15	Introduced in-line product quality monitoring or other process analysis system

W19	Other changes in operating practices

Inventory Control

W21	Instituted procedures to ensure that materials do not stay in inventory beyond shelf-life

W22	Began to test outdated material - continue to use if still effective

W23	Eliminated shelf-life requirements for stable materials

W24	Instituted better labeling procedures

W25	Instituted clearinghouse to exchange materials that would otherwise be discarded

W29	Other changes in inventory control

Spill and Leak Prevention

W31	Improved storage or stacking procedures

W32	Improved procedures for loading, unloading, and transfer operations

W33	Installed overflow alarms or automatic shut-off valves

W35	Installed vapor recovery systems

W36	Implemented inspection or monitoring program of potential spill or leak sources

W39	Other changes made in spill and leak prevention

Raw Material Modifications

W41	Increased purity of raw materials

W42	Substituted raw materials

W43	Substituted a feedstock or reagent chemical with a different chemical

W49	Other raw material modifications made

Process Modifications

W50	Optimized reaction conditions or otherwise increased efficiency of synthesis

W51	Instituted recirculation within a process

W52	Modified equipment, layout, or piping

W53	Use of a different process catalyst

W54	Instituted better controls on operating bulk containers to minimize discarding of empty containers

W55	Changed from small volume containers to bulk containers to minimize discarding of empty containers

W56	Reduced or eliminated use of an organic solvent

W57	Used biotechnology in manufacturing process

W58	Other process modifications

Cleaning and Degreasing

W59	Modified stripping/cleaning equipment

W60	Changed to mechanical stripping/cleaning devices (from solvents or other materials)

W61	Changed to aqueous cleaners (from solvents or other materials)

W63	Modified containment procedures for cleaning units

W64	Improved draining procedures

W65	Redesigned parts racks to reduce drag out

W66	Modified or installed rinse systems

W67	Improved rinse equipment design

W68	Improved rinse equipment operation

W71	Other cleaning and degreasing modifications

Surface Preparation and Finishing

W72	Modified spray systems or equipment

W73	Substituted coating materials used

W74	Improved application techniques

W75	Changed from spray to other system

W78	Other surface preparation and finishing modifications

Product Modifications

W81	Changed product specifications

W82	Modified design or composition of products

W83	Modified packaging

W84	Developed a new chemical product to replace a previous chemical product

W89	Other product modifications

### Methods Used to Identify Source Reduction Activities

For each source reduction activity, enter up to three of the following codes that correspond to the method(s) which contributed most to the decision to implement that activity.

T01	Internal Pollution Prevention Opportunity Audit(s)

T02	External Pollution Prevention Opportunity Audit(s)

T03	Materials Balance Audits

T04	Participative Team Management

T05	Employee Recommendation (independent of a formal company program)

T06	Employee Recommendation (under a formal company program)

T07	State Government Technical Assistance Program

T08	Federal Government Technical Assistance Program

T09	Trade Association/Industry Technical Assistance Program

T10	Vendor Assistance

T11	Other

## Section 8.11 Optional Pollution Prevention Information

### Barriers to Implementing Pollution Prevention Activities

B1	Insufficient capital to install new source reduction equipment or implement new source reduction activities/initiatives

B2	Require technical information on pollution prevention techniques applicable to specific production processes

B3	Concern that product quality may decline as a result of source reduction

B4	Source reduction activities were implemented but were unsuccessful

B5	Specific regulatory/permit burdens

B6	Pollution prevention previously implemented—additional reduction does not appear technically or economically feasible

B7	No known substitutes or alternative technologies

B8	Reduction does not appear to be technically feasible

B99	Other barriers

### Retired Codes

B8	Other barriers (replaced effective RY2018)

# Reporting the Waste Management of Metals

This chapter outlines how the TRI-MEweb reporting software restricts reporting for metals when the specific data element or waste management code is not applicable for a particular chemical. Below is a list of metals divided into four groups along with charts that help explain where quantities of these chemicals can and cannot be reported on the Form R using TRI-MEweb. In addition, there are charts that explain restrictions on reporting waste management codes for the toxic chemicals in each of the four groups. This chapter only shows where reporting is restricted in TRI-MEweb, it does not indicate every situation where a metal should not be reported in a specific section of the form. For example, TRI-MEweb does not restrict the reporting of most individually listed metal compounds as used for energy recovery (Sections 8.2 and 8.3) even though some of these chemicals do not have a heat value greater than 5000 British thermal units (Btu) and, thus, cannot be combusted for energy recovery. It is left to the facility to decide which of these toxic chemicals can be used for energy recovery.

| Parent Metals | Metal Compound Categories | Metals with Qualifiers | Individually Listed Metal Compounds |
| --- | --- | --- | --- |
| Antimony Arsenic Barium Beryllium Cadmium Chromium Cobalt Copper Lead Manganese Mercury Nickel Selenium Silver Thallium | Antimony Compounds Arsenic Compounds Barium Compounds Beryllium Compounds Cadmium Compounds Chromium Compounds Cobalt Compounds Copper Compounds Lead Compounds Manganese Compounds Mercury Compounds Nickel Compounds Selenium Compounds Silver Compounds Thallium Compounds Vanadium Compounds Zinc Compounds | Aluminum (fume or dust) Vanadium (except when in an alloy) Zinc (fume or dust) | Bis(tributyltin) oxide Triphenyltin hydroxide Triphenyltin chloride Molybdenum trioxide Thorium dioxide Asbestos (friable) Aluminum oxide (fibrous forms) Tributyltin fluoride Tributyltin methacrylate Titanium tetrachloride Boron trifluoride Metiram Boron trichloride Zineb Maneb Fenbutatin oxide Iron pentacarbonyl Ferbam C.I. Direct Brown 95 Osmium tetroxide Aluminum phosphide C.I. Direct Blue 218 |

## Section 5.3 Discharges to Receiving Streams or Water Bodies and Section 6.1 Discharges to Publicly Owned Treatment Works

The following chart indicates which metals can be reported as released to water in Section 5.3 or to POTWs in Section 6.1. Only zinc (fume or dust) and aluminum (fume or dust) are not reported in these sections because the fume or dust form of a toxic chemical cannot exist in water.

The release and other waste management information that you report for metal category compounds will be the total amount of the parent metal released and NOT the whole metal category compound. The metal cannot be treated because it cannot be destroyed. Thus, transfers of metals and metal category compounds for further waste management should be reported as a disposal. The applicable codes for transfers of metals and metal category compounds in wastewater to a POTW for disposal include P30, P31, P32, P33, P34, P35, and P36.

| Form R Section in Part II | Parent Metals | Metal Category Compounds | Metals with Qualifiers | Individually Listed Metal Compounds |
| --- | --- | --- | --- | --- |
| Section 5.3 - Discharges to receiving streams or water bodies | All | All | Vanadium (except when contained in an alloy) | All except Asbestos |
| Section 6.1- Discharges to POTWs | All | All | Vanadium (except when contained in an alloy) | All except Asbestos |

## Section 6.2 Transfers to Other Off-Site Locations

Any toxic chemical may be reported in Section 6.2. However, TRI-MEweb will not allow certain M codes to be used when reporting metals. The chart below indicates which M codes can be reported in Section 6.2 for the four groups of metals. Note that all disposal M codes other than M41 and M62 can be used for all toxic chemicals. Code M24 is only made available for the four groups of metals.

| Waste Management Code for Section 6.2 | Parent Metals | Metal Category Compounds | Metals with Qualifiers | Individually Listed Metal Compounds |
| --- | --- | --- | --- | --- |
| M41 and M62 (disposal codes for metals only) | All | All | Vanadium (except when contained in an alloy) | All except Asbestos |
| M56 and M92 (energy recovery codes) | None | None | None | All except Asbestos1 |
| M20 and M28 (recycling codes) | None | None | None | All |
| M24, M26 and M93 (recycling codes) | All | All | All | All |
| M40, M50, M54, (treatment codes) | None | None | All except Vanadium (except when contained in an alloy) | All |
| M61, M69, M95 (treatment codes) | Barium2 | Barium Compounds2 | Same as above | All |

1 Although TRI-MEweb does not restrict reporting of most individually listed metal compounds as transferred off site for energy recovery, only chemicals with a heat value greater than 5000 British thermal units that are combusted in a device that is an industrial furnace or boiler (40 CFR Section 372.3) should be reported as used for energy recovery.

2 The toxic chemical category barium compounds (N040) does not include barium sulfate. Because barium sulfate is not a listed toxic chemical, the conversion in a waste stream of barium or barium compound to barium sulfate is considered treatment for destruction (40 CFR Section 372.3).

## Section 7A: On-Site Waste Treatment Methods and Efficiency

TRI-MEweb allows any toxic chemical to be reported in Section 7A; however, it limits reporting in two ways. First, TRI-MEweb limits the treatment codes that can be reported based on the General Waste Stream Code selected. If a TRI-MEweb user selects General Waste Stream code “A – Gaseous”, all Waste Treatment Codes are made available. However, if a user selects from the remaining three General Waste Stream Codes (W - Wastewater, L - Liquid waste streams, or S - Solid waste streams), the “Air Emissions Treatment” Waste Treatment Codes are not made available. Second, the software restricts reporting for certain toxic chemicals with qualifiers. When reporting zinc (fume or dust) or aluminum (fume or dust) TRI-MEweb will not allow the user to select General Waste Stream Codes W-Wastewater and L-Liquid waste streams because the fume or dust form of a toxic chemical cannot exist in a liquid or water waste. For asbestos (friable) only S - Solid or A - Gaseous can be selected. When reporting hydrochloric acid (acid aerosols) or sulfuric acid (acid aerosols) only A - Gaseous can be selected.

## Section 7B: On-Site Energy Recovery Processes

| Energy Recovery Code for Section 7B | Parent Metals | Metal Category Compounds | Metals with Qualifiers | Individually Listed Metal Compounds |
| --- | --- | --- | --- | --- |
| U01, U02, U03 | None | None | None | All except Asbestos1 |

1 Although TRI-MEweb does not restrict reporting of most individually listed metal compounds as transferred off site for energy recovery, only chemicals with a heat value greater than 5000 British thermal units that are combusted in a device that is an industrial furnace or boiler (40 CFR Section 372.3) should be reported as used for energy recovery.

## Section 7C: On-Site Recycling Processes

Any chemical can be reported in Section 7C. However, certain waste management codes should not be reported for certain toxic chemicals. The chart below indicates which codes can be reported in Section 7C when using TRI-MEweb.

| Recycling Code for Section 7C | Parent Metals | Metal Category Compounds | Metals with Qualifiers | Individually Listed Metal Compounds |
| --- | --- | --- | --- | --- |
| H10 (this code is for metals only) | All | All | All | All |
| H20 | None | None | None | All |
| H39 | All | All | All | All |

## Section 8. Source Reduction and Recycling Activities

The chart below indicates which metals can be reported in Sections 8.2, 8.3, 8.6 and 8.7 of the Form R when using TRI-MEweb. Note that all toxic chemicals can be reported in Sections 8.1, 8.4, 8.5 and 8.8.

| Waste Management Activity | Parent Metals | Metal Category Compounds | Metals with Qualifiers | Individually Listed Metal Compounds |
| --- | --- | --- | --- | --- |
| Quantity used for energy recovery on site and off site (Sections 8.2 and 8.3) | None | None | None | All except Asbestos1 |
| Quantity treated for destruction on site and off site (Sections 8.6 and 8.7) | None except Barium2 | None except Barium Compounds2 | All except Vanadium (except when contained in an alloy) | All |

1 Although TRI-MEweb does not restrict reporting of most individually listed metal compounds as transferred off site for energy recovery, only chemicals with a heat value greater than 5000 British thermal units that are combusted in a device that is an industrial furnace or boiler (40 CFR Section 372.3) should be reported as used for energy recovery.

2 The toxic chemical category barium compounds (N040) does not include barium sulfate. Because barium sulfate is not a listed toxic chemical, the conversion in a waste stream of barium or barium compound to barium sulfate is considered treatment for destruction (40 CFR Section 372.3).