| Data Dictionary for Data Submissions Drinking Water National Environmental Public Health Tracking Network |  |
| --- | --- |
| Characteristic | Description |
| Data Source | Safe Drinking Water Act/State Drinking Water Information System (SDWA/SDWIS),or SDWIS-like Data System |
| Purpose | This data set contains the information needed to calculate Environmental Public Health Tracking (EPHT) measures of contaminants in public water supply for arsenic, disinfection byproducts, nitrates, atrazine, di(2-ethylhexyl) phthalate (DEHP), radium, tetrachloroethene (tetrachloroethylene) (PCE), trichloroethene (trichloroethylene (TCE), and uranium. Data are derived from state Safe Drinking Water Act databases. The data set consists of two tables: 1. PWS Inventory. 2. Drinking Water Quality Sampling Results. |
| Geographic Level | The smallest geographic unit to be represented in this data set is the county. |
| Restrictions | This is not a restricted access data set. Only registered users will have direct access to this data set via the national secure portal. |
| Instruction for Use | This data dictionary is a reference for creating the PWS Inventory data and Drinking Water Quality Sampling results data for submission. Refer to the dataset schemas for the order of variables and final data structure. |

Form Approved

OMB No. 0920-1175

Exp. Date 04/30/2020

CDC estimates the average public reporting burden for this collection of information as 100 hours per response, including the time for reviewing instructions, searching existing data/information sources, gathering and maintaining the data/information needed, and completing and reviewing the collection of information. An agency may not conduct or sponsor, and a person is not required to respond to a collection of information unless it displays a currently valid OMB control number. Send comments regarding this burden estimate or any other aspect of this collection of information, including suggestions for reducing this burden to CDC/ATSDR Information Collection Review Office, 1600 Clifton Road NE, MS D-74, Atlanta, Georgia 30333; ATTN: PRA (0920-0853).

| PWS Inventory Data Dictionary for Aggregate Data National Environmental Public Health Tracking Network |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Field Name | Field Description | Data Type | Code Scheme | Legal Values | Field Length | Optional |
| Latitude | Latitude in NAD83 decimal degrees describing approximate center of retail service area of water system | Integer | 00.0000000 to 90.000000, -99.99 for missing, -88.88 for Not Submitted | 00.0000000 to 90.000000, -99.99 for missing, -88.88 for Not Submitted | 10 | Required |
| LocationDerivationCode | Code describing how approximate latitude/longitude location was derived | Text | SA = Service area polygon centroid; MFL = Mean of 1 or more facility locations that are expected to be proximate to service area extent; PCS = GNIS coordinates for Principal City Served; GSH = The geocoded address of water system headquarters; PNS = GNIS coordinates for Principal County Served; O= Other (e.g. zip code, etc.); “-999” = Missing; “-888” = Not Submitted; (See “Appendix A. Service Area Location Derivation Guidance of the How-To Guide” for more guidance for deriving water system locations.) | Any of the codes in the Code Scheme column | 3 | Required |
| Longitude | Longitude in NAD83 decimal degrees describing approximate center of retail service area of water system | Integer | -180.000000 to 180.000000, -999 for Missing, -888 for Not Submitted | -180.000000 to 180.000000, -999 for Missing, -888 for Not Submitted | 11 | Required |
| PrimarySourceCode | Type of source | Text | GU = ground water under direct influence of surface water, GUP = purchased ground water under direct influence of surface water, GW = ground water, GWP = purchased ground water, SW = surface water, SWP = purchased surface water; U = Unknown; NS = Not submitted | Any of the codes in the Code Scheme column | 3 | Required |
| PrincipleCityFeatureID | Principal city, town or village Feature ID served by the CWS | Text | 9999999999; -999 = Missing; -888 = Not Submitted Feature ID can be obtained from: http://geonames.usgs.gov/domestic/download_data.htm | 9999999999, -999, -888 | 10 | Required |
| PrincipleCountyServedFIPS | Principal county FIPS served by the CWS | String | County FIPS, U = Unknown, NS = Not submitted | Valid county FIPS, U, NS | 5 | Required |
| PWSIDNumber | PWS Identifier | String | XXnnnnnnn | Nine character value consisting of the 2 letter state abbreviation followed by 7 numbers | 9 | Required |
| PWSName | Name of PWS | Text | Any, U = Unknown, NS = Not Submitted | PWS Name, U, NS, | 40 | Required |
| StateFIPSCode | State FIPS code | String | FIPS | Valid state FIPS code | 2 | Required |
| SystemPopulation | Permanent population uniquely served by the CWS | Integer | 10 - nnnnnnnn | 10-99999999 ” | 8 | Required |
| TotalConnections | Number of residential service connections | Integer | 1 - nnnnnnn | 1-9999999” | 7 | Required |
| YearAssociatedTo | Year that these data are associated with regards to sampling results | Integer | YYYY | 1999 through latest complete year | 4 | Required |
| YearPulled | Year that these data were pulled from state records | Integer | YYYY | 1999 through latest complete year | 4 | Required |

| Drinking Water Quality Samples Data Dictionary for Aggregate Data National Environmental Public Health Tracking Network |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Field Name | Field Description | Data Type | Code Scheme | Legal Values | Field Length | Optional |
| AggregationType | The type of summary operation performed (i.e. mean or max) for summary-level data. | Text | X = Mean Mean (for Annual and Quarterly data); MX = Maximum (For Annual Data ONLY) DO NOT SUBMIT FOR QUARTERLY DATA | X, MX | 3 | Required |
| AnalyteCode | USEPA Analyte code for required constituents (arsenic, nitrate, TTHM, HAA5, atrazine, PCE, TCE, DEHP, radium, and uranium) | String | 1005=Arsenic; 2050=Atrazine; 2456=HAA5; 2950=TTHM; 2039=DEHP; 1040=Nitrate; 2987=PCE; 2984=TCE; 4010=Combined Radium 226 & 228; 4006=Uranium (See How-To-Guide for converting gross alpha particle activity to U in ug/L) | Any of the codes in the Code Scheme column | 4 | Required |
| Concentration | Reported summary-level concentration or reported concentration of sample | Integer | nnnnnn.nnnn | Reported summary-level concentration or reported concentration of sample | 6,4 | Required |
| ConcentrationUnits | The analyte-specific units of summary-level measures and individual sample values as reported in the Concentration and DetectionLimit fields. Each analyte has a standard unit for this dataset. | Text | “ug/L” allowed only for (Arsenic, TTHM, HAA5, Atrazine, DEHP, PCE,TCE, uranium); “mg/L” allowed only for (Nitrate as nitrogen); “pCi/L” allowed only for (Radium) | Ug/L, mg/L | 6 | Required |
| DateSampled | Date of sample (sample-level data) or Date last sampled (summary-level data) | Text | YYYY-MM-DD | A valid date from 1/1/1999 through December 31st of the latest complete year (e.g. 2011-12-31). | 10 | Required |
| DetectionLimit | Sample detection limit | Integer | >0 if NonDetectFlag=1 -888, if NonDetectFlag=0 | >0, -888 | 3,6 | Optional |
| NonDetectFlag | Flag to indicate whether sample resulted in a detection or not | Integer | 1=Sample was a non-detect; 0=Sample was a detection | 0, 1 | 1 | Optional |
| NumNonDetects | The number of samples that were non-detections for summary-level data. | Integer | 0 – nnnn (nnnn must be greater than NumSamples) | 0 – 9999 | 4 | Required |
| NumSamples | The number of samples that were used in calculating the mean/max for a given analyte during a quarter or year. | Integer | 0 – nnnn | 0 – 9999 | 4 | Required |
| NumSamplingLocations | Number of compliance sampling locations available from which summary-level records were derived. | Integer | 0 – nnnn; -888 = Not Submitted | 0 – 9999, -888 | 4 | Required |
| PWSIDNumber | PWS identifier | String | XXnnnnnnn | Nine character value consisting of the 2 letter state abbreviation followed by 7 numbers | 9 | Required |
| SamplePointID | Sampling station identifier for sample-level records only. | Text | Character ID for sampling station; -999 = missing; -888 = Not Submitted | Valid Character ID for sampling station, -999, -888 | 20 | Optional |
| StateFIPSCode | State FIPS code | String | FIPS | Valid state FIPS code | 2 | Required |
| SummaryTimePeriod | Year or Quarter for summary-level data | String | YYYY for annual summarized values; YYYY-Q for quarterly summarized values (Allowed Only for AnalyteCodes 2050, 2456, 2950 and 1040) | 1999 through most current year, Quarters 1-4 (Ex: 2018, 2018-4) | 10 | Required |
| Year | Year | Integer | YYYY | 1999 – most current year | 4 | Required |